# (Your Business Name Here) – Safe Work Procedure

# PEDESTAL GRINDER

DO NOT use this machine unless you have been instructed

in its safe use and operation and have been given permission

## PERSONAL PROTECTIVE EQUIPMENT

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. | Hearing protection must be used when using this machine. |
| --- | --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. | Rings and jewellery must not be worn. |

## PRE-OPERATIONAL SAFETY CHECKS

Locate and ensure you are familiar with all machine operations and controls.

Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

Check workspaces and walkways to ensure no slip/trip hazards are present

Ensure that the wheels do not touch the work rest. Ensure the gap between wheel and rest is no more than 1.5mm.

Check that the wheels are running true and are not glazed or loaded.

Check for cracks in the wheel and report any you find.

## OPERATIONAL SAFETY CHECKS

Only one person may operate this machine at any one time.

Stand to the side of the wheels when starting up.

Let the wheels gain maximum speed before starting to grind.

Slowly move the workpiece across the face of the wheel in a uniform manner.

## ENDING OPERATIONS AND CLEANING UP

Switch off the machine when work completed.

Clean up and absorb any coolant spills immediately.

Leave the machine in a safe, clean and tidy state.

## POTENTIAL HAZARDS AND INJURIES

Hot metal.

Sparks.

Noise.

Sharp edges and burrs.

Hair/clothing getting caught in moving machine parts.

Wheels ‘run on’ after switching off.

Eye injuries.

## DON’T

Do not use faulty equipment. Immediately report any suspect machinery.

Do not hold workpiece with gloves, cloth, apron or pliers.

Do not grind non-ferrous metals.

Do not grind on the side of the wheel.

Do not hold small objects by hand.

Never leave the machine running unattended.

Do not bend down near the machine while it is running.

Never force the workpiece against a wheel.

This safe work procedure does not necessarily cover all possible hazards associated with this equipment and should be used in conjunction with other references. 
It is designed as a guide to be used to compliment training and as a reminder to users prior to equipment use.

ALWAYS refer to the manufacturer’s instructions when writing your own safe work procedure.