# (Your Business Name Here) – Safe Work Procedure

# ROUND HAY BALER

DO NOT use this machine unless you have been instructed

in its safe use and operation and have been given permission

## PERSONAL PROTECTIVE EQUIPMENT

## PRE-OPERATIONAL SAFETY CHECKS

Locate and ensure you are familiar with all machine operations and controls.

Ensure all guards are fitted, secure and functional. Do not operate if guards are missing or faulty.

Drive shafts and other moving parts must be guarded. Inverted U-shaped guards are inadequate.

Check for loose/missing nuts, bolts and screws Tighten and/or replace as needed.

Replace bent or missing pickup teeth to ensure effective feeding of material into the feed rolls.

Inspect all belts or chains for evidence of wear or breakage. Maintain belt tension according to the manufacturer’s recommendations.

Match belt lengths to prevent slippage that can cause blockages and heat build-up.

Before replacing a belt or chain, consult the manufacturer’s recommendations for instructions on securing the upper chain or moving load from the belt tension springs.

Check the slip clutch, roll scraper and rear gate latch to ensure they are adjusted and functioning according to the manufacturer’s recommendations.

Ensure the hydraulic hoses are clean, in good repair and hooked up correctly.

Ensure the hay baler is attached according to manufacturer’s recommendations.

Check that the twine feeding and cutting mechanisms are working properly and that the twine is in good condition.

## OPERATIONAL SAFETY CHECKS

When doing maintenance work, disengage the PTO, turn off the tractor, lock the tractor brakes, and remove the ignition key. Chock the baler wheels. Ensure all moving parts have completely stopped.

Operate the machine at a safe speed.

When operating over rough terrain or on hillsides, take care to avoid holes or obstacles that can tip a windrower or throw you from the machine.

When ejecting a bale, ensure the area behind the baler is clear before raising the tailgate. Do not eject on a downward slope as the bale may continue to roll.

If the tailgate must be opened for maintenance or repair, install a mechanical hydraulic cylinder lock out device to prevent accidental gate closing.

## CLEANING UP

Remove any foreign material from in and around moving parts.

Clean the baler to remove accumulated crop debris.

## POTENTIAL HAZARDS

Hair/clothing getting caught in moving machine parts.

Noise.

Thrown object hazards.

Crushing hazards.

## DON’T

Do not use faulty equipment.  Report suspect machinery immediately.

Never remove blockages or make adjustments until you have disengaged the PTO and shut off the tractor engine.

Never feed the material into the baler with your hands or feet.

Never handfeed or remove twine from the machine while it is running.

Never leave the tractor seat with the engine running, even if the PTO is disengaged.

This safe work procedure does not necessarily cover all possible hazards associated with this equipment and should be used in conjunction with other references. 
It is designed as a guide to be used to compliment training and as a reminder to users prior to equipment use.

ALWAYS refer to the manufacturer’s instructions when writing your own safe work procedure.